-----------------------------------------------------------------

TABLE OF CONTENTS                                 PAGE

-----------------------------------------------------------------

INSTALLING SIGNTALK ON YOUR PC................  1

BASIC PRINCIPLES OF OPERATION.................  2

HOW TO USE THE EDITOR.........................  3

SENDING YOUR PROGRAM TO THE SIGN..............  4

STRUCTURING YOUR PROGRAM......................  4-5

PROGRAMING....................................  6-11

STREAMING.................................  6

SINGLE LINE SHORT MESSAGE ENTRY...........  7

SINGLE LINE HIGHLIGHTING EFFECTS..........  8

SINGLE LINE EXIT COMMANDS.................  8

MULTI-LINE MESSAGE ENTRY..................  9

TIMING CONTROL COMMANDS...................  9

FREEZING TEXT ON SCREEN...................  9

LETTER STYLE AND SPANISH COMMANDS.........  10

CLOCKS, SPECIAL STAND-ALONE EFFECTS.......  10

GRAPHICS..................................  11

SETTING THE TIME AND DATE.....................  12

STORING MULTIPLE PROGRAMS ON YOUR DISK........  13

SIGNTALK DISK FILES INFORMATION...............  14

SAMPLE DEMOCOPY PROGRAM.......................  15-18

1.

-----------------------------------------------------------------

SUNRISE SYSTEMS PC BASED LED SIGN CONTROL PROGRAM

-----------------------------------------------------------------

This manual shows you how to use your PC and the program disk

supplied to control your Sunrise Systems, Inc.  EMX-2000 LED sign.

HOW TO INSTALL THE SOFTWARE ON YOUR COMPUTER:

The first thing to do is to make a backup copy of your disk and

keep it in a safe place.  If your computer has a hard disk you

should make a directory for this system and copy the files from

the floppy into it.  A typical sequence to do this starting at the

C:> prompt would be

MKDIR \SUNRISE

CD \SUNRISE

COPY A:*.*/V

If your computer only has a floppy disk drive there is no need to

perform any installation if it can boot up DOS from ROM without

a floppy disk in the drive.  If it requires a system floppy disk

to be in at boot up, you will have to format a new system disk

and copy the files from the Sunrise Systems distribution disk to

the new system disk.  If your computer has two floppy drives there

is no need to perform any installation.

STARTING UP THE COMPUTER TO RUN THE SOFTWARE:

If your computer has a hard disk drive you should should wait to

see the C:> DOS prompt and then change directories with the

command:  CD \SUNRISE

If your computer only has a single floppy disk drive and if it can

boot up DOS from ROM without a disk installed, you should just

turn it on and wait to see the C:> prompt.  Then insert the

floppy disk and enter A: to get the A:> prompt.

If your computer only has a single floppy disk drive and cannot

boot DOS without a disk in the drive, then you must put a system

disk with the Sunrise Systems software on it into the drive before

turning the computer on.

If your computer has two floppy drives but no hard disk drive,

you should boot the computer up with a system disk in drive A:

until you get the A:> prompt.  Then put the disk with the

Sunrise Systems software in drive B and enter  B:  to get the

B:> prompt.

You should hit the CAPS LOCK key to turn CAPS LOCK on for most

operations with this software.

2.

BASIC PRINCIPLE OF OPERATION

The software includes a text editor that allows you to view

and optionally edit a plain text file that contains your

messages and your instructions on how you want the sign to

show them.  The name of the disk file that holds your active

sign program is called  SIGNCOPY.  To edit your program you

simply give the command at the DOS prompt:  EDIT SIGNCOPY

Further on we will tell you more about editing.  If you prefer

another editor over the one supplied you may use it as long

as it produces plain ASCII text files.

Once you have edited your SIGNCOPY and have returned to DOS,

you can run a program called LOADSIGN to send your SIGNCOPY

to the sign.   That's all there is to it for virtually all

normal operations.  There is also a more comprehensive

program supplied called SIGNTALK which allows interactive

communication with the sign through the PC keyboard and

screen.  This may be used to set the time and date in the

sign or for other special purposes.

3.

HOW TO USE THE EDITOR

At the DOS C:> or A:> prompt you must enter the command

EDIT SIGNCOPY to start up the editor.  After a few seconds

you will see the text of your SIGNCOPY on the screen.  You

should hit the INSERT key to see the INSERT flag come on

in the lower right corner of the screen.  This means that things

which you type will be inserted rather than overstriking the

character that was there already.  You can use the cursor

arrow keys to move up down left and right.  Whatever you

type will be inserted at the cursor.  The DELETE and BACKSPACE

keys erase text forward and backward.  (CTRL)X erases a whole

line.  (CTRL)Z erases to the end of the line.

HOW TO GET OUT OF THE EDITOR

When you are done editing, you should hit the ESC key.  You

will see a command line at the bottom of the screen.  If you

want to make your changes permanent, type EX and then hit the

ESC key twice.  The ESC key will show as a dollar sign on

your screen.  You will go back to the DOS prompt.  If you

want to forget your edits, instead of EX type EQY and then

hit the ESC key twice.  You will return to DOS and your

SIGNCOPY will be as it was before you started to edit.  Just

in case you make a big mistake and do an EX exit with bad

copy and you find out after sending it to the sign, you can

recover because the EDIT program always makes a backup copy

of your most recent SIGNCOPY called SIGNCOPY.BAK which you

can get back with the DOS command: COPY SIGNCOPY.BAK SIGNCOPY

4.

HOW TO SEND YOUR PROGRAM TO THE SIGN

Connect the cable from your sign to the PC's COM1 serial port.

Be slow and careful handling the cable.  Avoid bending or

breaking pins on the PC or the sign connectors.  Be careful

to avoid discharging static sparks into the cable.  If you are

on a carpet or near furniture that makes sparks, discharge

yourself to ground first before handling the cable.  Be sure

your sign is powered on.  At the DOS prompt give the command

LOADSIGN.  The PC will load your sign automatically.  First

it will send a processed version of your SIGNCOPY and then

it will send a large block of special code that supports the

operations of all the formatting commands.  When the send is

done the program will start to run.  You may disconnect the

cable whenever you are ready.

HOW TO FORMAT YOUR SIGNCOPY

Messages are entered into SIGNCOPY using the editor in the

order that the sign will present them.  SIGNCOPY contains

a mixture of message copy which you want to display with

formatting commands that tell the sign how you want each

message to move.

MESSAGE COPY can be of two basic forms, long and short.  Short

messages must be short enough to fit on the sign all at once.

Long messages can only be displayed by streaming them across

the sign.  In either case the message text is entered starting

at the left end of a line.  It usually should be in all capitals

since that is the best appearance on the sign.  Long messages

can continue on multiple lines with lines broken at word

boundaries.  Do not put spaces at the end of lines since long

messages on multiple lines will have a space automatically

placed at each line break.

FORMATTING COMMANDS are of several varieties.  They all begin

with an = sign at the left end of a line.  Most of them will

perform some action but a few of them have more subtle actions

like selection of typefaces or introduction of pauses.

COMPLETE MESSAGES generally consist of MESSAGE COPY followed

by one or more FORMATTING COMMANDS.  There are some canned

effects possible that are called up with just a formatting

command that do not accept any message copy.

SEPARATION OF COMPLETE MESSAGES from each other is performed

by one or more blank lines.

5.

An example of a complete message follows:

WELCOME TO OUR NEW BRANCH OFFICE...  WE LOOK FORWARD

TO SERVING YOU HERE FREQUENTLY...

=BOLD

=STREAM

The message text comes first.  The =STREAM is the formatting

command that actually makes the message move.  The =BOLD

formatting command selects the typeface.

This command controls the style and visual appearance of the

message when =STREAM moves it.  Because the =BOLD

exerts a controlling influence on the =STREAM

action command, it must precede the =STREAM action command.

Any commands such as =BOLD which control style in any

way, such as typeface or speed must come before the

action commands which they are to influence.

At the end of this manual is a listing of the DEMOCOPY file

which illustrates usage of many of the formatting commands.

6.

******** PROGRAMMING COMMANDS ********

-----------------------------------------------------------------

STREAMING ACTION COMMANDS

Streaming commands take messages of any length.

=STREAM          RUNS MESSAGE FROM RIGHT TO LEFT ACROSS THE SIGN

USING THE CHOSEN FONT

=BIG-STREAM       SAME AS ABOVE - CHOSEN FONT IS DOUBLE WIDE

=PHONE-STREAM     PHONE PULLS MESSAGE

=CAR-STREAM       CAR PULLS MESSAGE

=TRUCK-STREAM     SEMI PULLS MESSAGE

=BOAT-STREAM      YACHT PULLS MESSAGE

=LINER-STREAM     CRUISE LINER PULLS MESSAGE

=CASH-STREAM      DOLLAR SIGNS PULL MESSAGE

=PLANE-STREAM     BI-PLANE W/ SPINNING PROPELLER PULLS MESSAGE

=JET-STREAM       JET PULLS MESSAGE

=HOUSE-STREAM     HOUSE PULLS MESSAGE

=BILL-STREAM      GREENBACK PULLS MESSAGE

=HEARTS-STREAM    HEARTS PULL MESSAGE

=STARS-STREAM     STARS PULL MESSAGE

=SANTA-STREAM     SANTA W/ BOUNCING REINDEER PULL MESSAGE

(No additional graphics may be used with SANTA)

7.

-----------------------------------------------------------------

SINGLE LINE SHORT MESSAGE ENTRY ACTION COMMANDS

Message copy used with these must be short enough to fit on

the LED sign all at one time.  The number of characters that

will fit depends on the length of your sign and the currently

selected letter style.  If you are adding a [GRAPHIC] to

a single line message, be sure to use the Character-Spaces-Used

figures supplied in the graphics listing when counting your

characters.  These action commands are affected also by the

current speed setting.  After a message enters it automatically

waits briefly before the program moves on.  You may use

the AUTOHOLD functions described later on to control the amount

of time that the message holds after entering.

=IN              TEXT APPEARS IMMEDIATELY

=IN-ABOVE        TEXT ENTERS FROM ABOVE

=IN-BELOW        TEXT ENTERS FROM BELOW

=IN-LEFT         TEXT ENTERS FROM LEFT

=IN-RIGHT        TEXT ENTERS FROM RIGHT

=IN-CURTAINS     CURTAINS CLOSE THEN OPEN TO REVEAL TEXT

=IN-CENTER       TEXT ENTERS FROM CENTER TO ENDS

=IN-ENDS         TEXT ENTERS FROM ENDS TO CENTER

=IN-TYPE         TEXT TYPES IN FROM LEFT TO RIGHT

=IN-SPARKLE      TEXT APPEARS WITH SPARKLES BEHIND IT

=IN-RADAR        TEXT APPEARS WITH RADAR WAVES BEHIND IT

=IN-FRAME        TEXT APPEARS WITH FRAME BEHIND IT

=IN-FLASH        TEXT APPEARS FLASHING ON AND OFF

=IN-ZOOM         TEXT APPEARS WITH DOTS FLYING BEHIND IT

=IN-LAYER        TEXT STACKS IN LINE BY LINE

8.

----------------------------------------------------------------

SINGLE LINE SHORT MESSAGE HIGHLIGHTING ACTION COMMANDS

These commands can used after one of the single line entry

commands described above has already brought a short message

onto the screen. These commands will cause the existing message

on the screen to do some action that will draw more attention.

You will notice that many of the commands below are also single

line entry commands- this means that you can use several

commands in a row for added highlighting.

Do not supply any new message copy prior to using one of these

commands as a highlighting tool.

=SPARKLE         SPARKLES IN BACKGROUND BEHIND DISPLAYED TEXT

=TWINKLE         DOTS TWINKLE AROUND LETTERS

=FLASH           FLASHES DISPLAYED TEXT

=RADAR           RADAR WAVES RUN CENTER TO ENDS BEHIND TEXT

=WAVE            WAVES DISPLAYED TEXT BACK AND FORTH

=FLIP            FLIPS TEXT BACKWARDS END OVER END TOP TO BOTTOM

=MOVE<           VISIBLY SHIFTS DISPLAYED TEXT TO SCREEN'S LEFT

=MOVE>           VISIBLY SHIFTS DISPLAYED TEXT TO SCREEN'S RIGHT

----------------------------------------------------------------

SINGLE LINE SHORT MESSAGE EXIT ACTION COMMANDS

These commands can used after one of the single line entry

commands described above has already brought a short message

onto the screen.  They will cause the existing message on

the screen to leave the screen.  Do not supply any new message

copy prior to using one of these.

=OUT             DISPLAYED TEXT DISAPPEARS IMMEDIATELY

=OUT-BELOW       DISPLAYED TEXT EXITS TO BOTTOM

=OUT-ABOVE       DISPLAYED TEXT EXITS TO TOP

=OUT-LEFT        DISPLAYED TEXT EXITS LEFT

=OUT-RIGHT       DISPLAYED TEXT EXITS RIGHT

=OUT-ENDS        DISPLAYED TEXT SPLITS OFF ENDS TO CENTER

=OUT-CENTER      DISPLAYED TEXT SPLITS OFF CENTER TO ENDS

=OUT-TNT         DISPLAYED TEXT EXPLODES UP

=OUT-LAYER       DISPLAYED TEXT LIFTS OUT LINE BY LINE

=OUT-GRAB>       CLAW GRABS TEXT PULLS IT OFF TO RIGHT

=OUT-GRAB<       CLAW GRABS TEXT PULLS IT OFF TO LEFT

=OUT-PAC>        DISPLAYED TEXT EATEN LEFT TO RIGHT

=OUT-PAC<        DISPLAYED TEXT EATEN RIGHT TO LEFT

9.

-----------------------------------------------------------------

MULTI-LINE MESSAGE ENTRY ACTION COMMANDS

These action commands can deliver a long message to the sign

as a series of short messages.  Each line of the message

will be shown separately in succession.  As with the isolated

single line messages you must limit the length of each line

to what will fit on your sign.  This will vary by the length

of your sign and the width of the selected type style.

=ROLL-LINES       EACH LINE ROLLS UP

=TYPE-LINES       EACH LINE TYPES ON AND THEN ROLLS UP

=WIPE-LINES       EACH LINE IS WIPED ON BY FLYING BALL

=DISSOLVE-LINES   EACH LINE IS DISSOLVED ONTO THE SCREEN

As with the isolated single line message entry actions, these

actions are affected by the speed and the AUTOHOLD setting.

----------------------------------------------------------------

TIMING CONTROL COMMANDS

=SLOW            THESE FOUR COMMANDS SET SPEED FOR ACTION COMMANDS

=MEDIUM

=FAST

=FASTER

=PAUSE           WAIT NOW FOR 1 SECOND

=WAIT            WAIT NOW FOR 2 SECONDS

=LONGWAIT        WAIT NOW FOR 5 SECONDS

=AUTOHOLD.5      CAUSES SINGLE FRAME ENTRY ACTIONS TO HOLD 1/2 SEC

=AUTOHOLD1       CAUSES SINGLE FRAME ENTRY ACTIONS TO HOLD 1 SEC

=AUTOHOLD2       CAUSES SINGLE FRAME ENTRY ACTIONS TO HOLD 2 SEC

=AUTOHOLD3       CAUSES SINGLE FRAME ENTRY ACTIONS TO HOLD 5 SEC

----------------------------------------------------------------

SCREEN FREEZING COMMANDS

=FREEZE          FREEZES ALL NONBLANK COLUMNS OF DISPLAYED TEXT

SO THAT SUBSEQUENT ACTIONS LIKE STREAMING WILL

APPEAR TO BE BEHIND THE FROZEN COLUMNS

=UNFREEZE        RELEASES FROZEN COLUMNS

1O.

-----------------------------------------------------------------

LETTER STYLE FORMATTING COMMANDS

=THIN            THIN LETTERS (default)

=BOLD            BLOCK LETTERS

=WIDE            WIDE LETTERS

=BOLDWIDE        WIDE THICK LETTERS

=SUPERWIDE       WIDER THICK LETTERS

CHARACTERS PER SCREEN - BY SIGN LENGTH IN COLUMNS

---------------------------------------------------------------------

Style - Columns:  80      96     112     128     192     224     256

---------------------------------------------------------------------

=THIN ............13......16......18......21......32......36......42

=BOLD ............10......12......14......16......24......28......32

=WIDE .............6.......8.......9......10......16......18......21

=BOLDWIDE .........6.......8.......9......10......16......18......21

=SUPERWIDE ........5.......6.......7.......8......12......14......16

---------------------------------------------------------------------

SPANISH CHARACTERS - using the =SPANISH instruction

When entering spanish text the are three characters that aren't

available on your keyboard.  Three seldem used characters have

been chosen to replace them which are:

@ (at sign)     stands for

\ (back slash)  stands for

_ (underline)   stands for

@QUIEN ERES TU SE_OR?  @BIEN?  \BUENO!

=BOLD

=SPANISH

=STREAM

Note that the =SPANISH command follows the letter choice

command.  =SPANISH tells the sign to look for the special

characters in your text and convert them.  The reason =SPANISH

ALWAYS FOLLOWS the letter choice command is so that the

converted letters will be the correct size.

----------------------------------------------------------------

STAND ALONE CANNED EFFECTS

Action commands in this category do not require or accept any

associated message.

=PLAINCLOCK      TIME, DAY, DATE APPEAR, HOLD, EXIT TO TOP

=FANCYCLOCK      TIME, DAY, DATE APPEAR, HOLD, THEN EXIT

VIA GRAB>, GRAB<, AND PACMAN>

11.

-----------------------------------------------------------------

GRAPHICS

The graphics listed below can be inserted into your text

wherever you want them just by typing them in as shown.

For instance:

CALL NOW TO SIGN UP FOR THE CARIBBEAN CRUISE

CONTEST [LINER]     ALL ENTRIES MUST BE PHONED IN BY 5 PM

MONDAY...      CALL 555-1234 NOW...

=BOLD

=PHONE-STREAM

The above copy would stream by with the graphic of a phone

pulling the message with the graphic of an oceanliner

following the the word contest.

Only one graphic can be added to any given message, but the same

graphic can be added more than once within that message. [LINER]

could have been used several times in the above message.

The limit of only one graphic per message does not apply to the

either [TM] or [R].  Either of these graphics can be

used where needed even if another graphic is already in use.

-----------------------------------------------------------------

GRAPHIC         DESCRIPTION              CHARACTER SPACES USED

-----------------------------------------------------------------

[TM]            SMALL TM                             2 CHARS

[R]             CIRCLE-R REGISTERED MARK             2

[CAR]           LIMO                                 5

[TRUCK]         SEMI                                 5

[JET]           AIRLINER                             6

[PLANE]         BI-PLANE                             4

[PHONE]         TELEPHONE                            3

[PC]            PC TERMINAL                          2

[BOAT]          YACHT                                3

[LINER]         OCEANLINER                           7

[FISH]          FISH                                 3

[BILL]          GENERIC GREENBACK                    4

[$$$]           3 LARGE DOLLARSIGNS                  6

[HOUSE]         LITTLE HOUSE (prairie not incl.)     2

[STAR]          5 POINTED STAR                       2

[HEART]         VALENTINE STYLE (no arrow)           2

[ARROW-UP]      ARROW POINTING UP                    2

[ARROW-D]       ARROW POINTING DOWN                  2

[ARROW-R]       ARROW POINTING RIGHT                 2

[ARROW-L]       ARROW POINTING LEFT                  2

12.

HOW TO SET THE TIME AND DATE ON YOUR SIGN

The sign has a crystal clock that is always running even when

the power is off because it is protected by a battery.  It

will drift with time however and there are changes to daylight

and standard time and leap years to be accommodated.  To

access the sign to set the time and date you must do the

following:

Connect the cable from the sign to the computer as you

normally do.

Reboot the computer from a power off condition.

At the DOS prompt enter the command  SIGNTALK

When you see the menu enter L to talk to the sign

Hold down the CTRL key and hit D to break into the sign editor.

You should see a flashing cursor.  If you do not see this,

try typing YS to show editing.  YH will restore hidden editing

later if you like.

To set the time type TT followed by four digits in 24 hour mode.

For example TT1530 sets the clock to 3:30 PM.  The clock will

run continuously until you hit the Enter key to get back to

the flashing cursor.

To set the date type TD followed by two digits of month and two

digits of date.  For example, TD1231 sets it for December 31.

You should hit the Enter key afterward to get back to the

flashing cursor.

To set the day of the week, type TW followed by a digit 1-7

where Sunday is 1 and Saturday is 7.  You should hit the

Enter key afterward to get back to the flashing cursor.

Type P to start the program running.

Hit the ESC key to get back to the SIGNTALK menu.  Then hit Q

to quit to DOS.

13.

HOW TO KEEP MULTIPLE PROGRAMS FOR THE SIGN ON DISK

In this manual we only speak of editing the active sign

program which is kept in SIGNCOPY.  You can easily keep many

programs on the disk however by using the DOS COPY command.

To save the current SIGNCOPY as an alternate program named

WEEKEND just enter the DOS command:  COPY SIGNCOPY WEEKEND

To reactivate that program at a later time just enter the

DOS command:  COPY WEEKEND SIGNCOPY

You can also edit the alternate programs directly as in

the command EDIT WEEKEND rather than copying them into

SIGNCOPY for editing and then copying them back out again.

If this is done a great deal you will get a large number of

.BAK files built up on the disk over time which should be

purged from time to time with the command  ERASE *.BAK  to

make more disk space and clean up the directory.

You may wish to use the DOS command DIR/W to see all the

files in wide page form if you get many of them on the disk.

14.

FOR REFERENCE ONLY...   THE FILES ON THE DISTRIBUTION DISK ARE:

SIGNFX.X2S   This file is the operating software for the

system and contains the effect-specific programs

needed to display your messages.  This file is not

normally accessed by the user and is created for you

by SUNRISE SYSTEMS. It is where the color choice,

letter choice, graphics, entry effects and specially

tailored "canned" effects reside.

SIGNCOPY     This file holds your sign message text and your

instructions on how the sign should show them.

SIGNCOPY.BAK This is a backup copy of SIGNCOPY which EDIT makes

to let you recover from a catastrophic editing error.

DEMOCOPY     This is a sample demo copy of SIGNCOPY that shows

examples of various sign display techniques.

LOADSIGN.EXE This is the program that loads your SIGNCOPY to the

sign.

SIGNTABL.TXT This file is a conversion table used by

LOADSIGN.EXE .  It contains a list of of the user

available commands and their equivilent in program-

ming language.  LOADSIGN reads SIGNCOPY and

translates it into a sign-executable program which

it sends to the sign.

SIGNTALK.EXE This program contains a MENU that allows interactive

access to the sign from the PC.  It can be used to

do direct programming on the sign, to change the

time and date, and to save and recall programs on

the disk.

EDIT.COM     This is the text editing program.

SIGN.DOC     This is the text of this manual.

15.

******** SAMPLE DEMO PROGRAM ********

This program is on your disk as DEMOCOPY.  To see it run use

the DOS command  COPY DEMOCOPY SIGNCOPY  and then LOADSIGN.

If the program that was in SIGNCOPY to begin with is valuable

be sure to copy it to another file before overwriting it with

DEMOCOPY.

HELLO

=SUPERWIDE

=IN-FRAME

=SPARKLE

=OUT-PAC>

WELCOME TO THE SUNRISE SYSTEMS INC SIGNTALK[TM] [PC] DEMO PROGRAM

=PLANE-STREAM

@QUIEN ERES TU SE_OR?    @BIEN?    \BUENO!

=BOLD

=SPANISH

=STREAM

=FANCYCLOCK

OFFICE LOBBIES

=BOLD

=IN-SPARKLE

=OUT-ABOVE

USE SIGNTALK TO

WELCOME GUESTS

AND KEEP THEM

INFORMED WHILE

THEY WAIT FOR

THEIR APPOINTMENT

=AUTOHOLD1

=DISSOLVE-LINES

PICK UP OUR LATEST{ABC NEWSLETTER}

TAKE NOTE OF OUR NEW{LOWER}RESELLERS PRICE LINE ON PAGE 3!

SEND IN FOR THE [PC] ABC VIRUS-HUNTER[TM] FREE TO ALL REPS

=BOLD

=ACCENT-STREAM

16.

BANKS!

=BOLD

=IN

=TWINKLE

=MOVE<

=FREEZE

BANKS CAN USE THE SIGNTALK[TM] SYSTEM TO DIRECT THEIR CUSTOMERS

=THIN

=STREAM

=UNFREEZE

APPLY FOR AN

AUTO LOAN [CAR]

OR CREDIT CARD

INSTANTLY!

=THIN

=AUTOHOLD1

=ROLL-LINES

JUST USE THE SPECIAL PHONE [PHONE] TO TALK TO A BANK SPECIALIST...

YOU CAN ALSO -   APPLY FOR CREDIT...  STOP PAYMENT ON A CHECK...

PLUS MUCH MORE!

=BOLD

=PHONE-STREAM

GET OUR [HOUSE]

CURRENT LOW

MORTGAGE RATES

JUST CALL

555-1234

=BOLD

=AUTOHOLD1

=ROLL-LINES

USE THE INFORMATION TERMINAL [PC] TO DISCOVER THE POTENTIAL SAVINGS OF

REFINANCING YOUR MORTGAGE...

LEARN MORE ABOUT PRODUCTS AND SERVICES...

RECEIVE UP-TO-DATE ACCOUNT INFORMATION...  EVEN REORDER CHECKS...

=THIN

=HOUSE-STREAM

17.

MANUFACTURING

=BOLD

=IN-CURTAINS

=FLIP

=OUT-TNT

GET YOUR MESSAGE

OUT QUICKLY TO

YOUR EMPLOYEES...

IN LUNCH ROOMS,

HALLWAYS, GATES,

& PLANT FLOORS

=THIN

=AUTOHOLD.5

=TYPE-LINES

FISH DELIVERY TRUCKS MUST GO TO [FISH] DOCK 86 [FISH]

=BOLD

=TRUCK-STREAM

21 SAFEDAYS

=BOLD

=IN-FRAME

=PAUSE

IN A ROW!

=BOLD

=IN-FRAME

=PAUSE

GOOD WORK!

=BOLD

=IN-FRAME

=PAUSE

KEEP IT GOING

=THIN

=IN-FRAME

=SPARKLE

=OUT-TNT

18.

SCHOOLS

=BOLD

=IN-ABOVE

=RADAR

=OUT-GRAB>

L.E.D. SIGNS MAKE GREAT CLASS GIFTS [HEART][HEART][HEART]

=BOLD

=STARS-STREAM

[ARROW-UP][ARROW-UP][ARROW-UP]

REGISTRATION

[ARROW-UP][ARROW-UP][ARROW-UP]

=BOLD

=STREAM

[ARROW-L] CHEERLEADER TRY OUTS [ARROW-L]

=THIN

=STREAM

[ARROW-R] PTA MEETING [ARROW-R]

=WIDE

=STREAM

FIELDTRIP CARPOOLERS [ARROW-D] MEET AT NOON IN LOWER PARKING AREA 3

=THIN

=CAR-STREAM

SIGNTALK[TM]

=SUPERWIDE

=BIG-STREAM

SUNRISE

=BOLDWIDE

=IN-LAYER

=PAUSE

=OUT-LAYER

SYSTEMS

=BOLDWIDE

=IN-LAYER

=PAUSE

=OUT-TNT

THANK YOU!

=BOLD

=IN-ZOOM

=OUT-UP

CALL 617-826-9706 FOR MORE INFORMATION ON SIGNTALK[TM] [PC]

=THIN

=PHONE-STREAM