Table of Contents:

Introduction	2

Disclaimer	2

PARTS LIST	3

SOME REQUIREMENTS	3

Tools	3

Before you start	3

ASSEMBLY PROCEDURE	4

Soldering all components	4

HOW TO CONNECT TO AN ILDA DB25 CONNECTOR	4

TESTING THE BOARD	5

Errata	6

Suggestions for future improvements	6

# Introduction

Congratulations for purchasing this kit, I hope you will enjoy building it, and maybe you will even learn a bit more about electronics.  But mostly, I wish you many years of fun using it.  Despite all my efforts to offer quality products, there’s still a possibility that some errors remain unnoticed, or things overlooked.  I am always happy to receive feedback and positive critics where the intent is to help me improve both design and documentation in the future.

I also invite you to go see my web page, where this kit’s design has come from, along with more to come:  http://norm.servehttp.com

Norm

eKit designer

# Disclaimer

I will not be held responsible for property damage or personal injuries.  I cannot guarantee the proper functionality or performance of this kit.   This DAC board is one component of a laser show project, and even though it does not directly represent a danger for the eyes in itself, I still feel that it is important to remind you the following:  Lasers and even their reflections can be dangerous and should never be aimed at people’s eyes.  Always wear appropriate safety goggles, considering the laser’s wavelength you are working with.

# PARTS LIST

When you receive your kit, check if everything is there.

1X	Schematic		May be sent by email

1X	PCB

1X	DB25			Male connector

1X	7805			Fixed +5V voltage regulator

2X	DAC0832LCN	8 bits double buffered DACs

1X	TL074P		Quad low noise opamps

2X	20 pins DIP sockets

1X	14 pins DIP sockets

2X	10K			resistors

4X	20K			resistors

4X	0.1uF	(or 0.01uF)	capacitors

1X	1uF			electrolytic capacitor

1X	33uF			electrolytic capacitor

# SOME REQUIREMENTS

## Tools

- Soldering iron with fine tip

- DC Voltmeter

- Cutters

- Oscilloscope is nice to have, but not required

## Before you start

Some important things you must know:

- As stated in the Errata section below, the silk screen has the power supply pins identified backwards.  In other words, +15V is actually -15V and vice versa.

- DB25 connector must be soldered on solder side, not components side.

- Outputs X+ and Y+ are single ended (not differential), and can vary from +10V to -10V, relative to ground (X- and Y-).  If you are using this DAC with galvo drivers that have differential inputs (mine are not), just tie their X- and Y- inputs to ground, and this will make them compatible with this DAC.  Of course, their GND must be tied to this DAC’s GND also.

- X+/X- are for the horizontal output signal, Y+/Y- is for vertical

- G+/G- is the Green TTL modulation signal output.  R+/R- is the same but for Red.

- Currently (Feb 2007), only the green blanking is supported by NLS version 1.0.

- About IC2 and J2:  These are not used presently, but I added them for future expansion.  Eventually, these output pins could be used for RGB blanking, motor control, relay and solenoid activation.

# ASSEMBLY PROCEDURE

## Soldering all components

- C1 and C2 must be soldered taking in account their polarity, as in Picture 1.

- DB25 connector must be soldered on solder side, not components side.

+

+

-

-

Picture 1

# HOW TO CONNECT TO AN ILDA DB25 CONNECTOR

To write…

# TESTING THE BOARD

- Download NLS (Norm's Laser Show) software from my web site:  http://norm.servehttp.com

- Unzip into a location of your choice, but be sure to keep the folder architecture (Turn ON the "Use Folder Names" checkbox)

- Start the program and ensure that the DAC settings are set to DAC0832.  This is done in File/Settings menu.

- With DAC board powered OFF, connect the LPT port of your computer to the DAC board's DB25 connector using a straight parallel male to parallel female cable.  Note that if you go buy one, don't just ask for a printer cable because you will end up with a Centronics connector at one end!  Ensure that you ask for a parallel to parallel cable, and inspect it carefully to see if both ends have DB25 connectors.

- Connect both outputs of the DAC board to the galvo drivers' input.  You may want to connect to an oscilloscope in X/Y mode instead, especially if unit is tested for the first time after assembling.

- If using a driver with galvos, set both axis size pots to zero, and start the ILDA test pattern (ildatest.ild).

- Gradually increase the level just a bit (~20% max), and observe output.  At that point, if you still don't see any signal at outputs, remove power and find/fix the problem.   Carefully inspect board for broken conductors, solder bridges, excess flux, shorted capacitors, etc.

# Errata

- Silk screen has power supply pins identified backwards, i.e. +15V is actually -15V and vice versa.

- C1 and C2 silk screen lack the polarization info.  See “Soldering all components” section for details.

# Suggestions for future improvements

- Would be good to add fuses for +15V and -15V power supply inputs.

Anything else?  Send them to me at legato@videotron.ca

Thanks again, and have fun!

Norm, eKit

legato@videotron.ca